Session #2:  DST set up   Participant:____________

Each skill demonstrated by the participant will be ranked on a scale of 1-4:

1 = needs more practice

2 = acceptable, still continue to practice

3 = good, minor refinement necessary for better flow and confidence

| Techniques | Score | Comments |
| --- | --- | --- |
| Set up of work space |  |  |
| The is workspace set up properly |  |  |
| Work is always moving in the direction of clean to dirty |  |  |
| Aseptic technique |  |  |
| Good aseptic technique is understood and demonstrated |  |  |
| Uncapping and dexterity |  |  |
| Good dexterity is exercised in removing and replacing caps |  |  |
| Tubes are held comfortably while working |  |  |
| Dilution preparation |  |  |
| Seed tubes are vortexed and allowed to settle 20 min |  |  |
| Dilutions are prepared and inoculated correctly |  |  |
| Use of transfer pipettes |  |  |
| Inoculum is taken for the top 1/3 of the tube, tip of pipette does not go past the bottom of the label |  |  |
| Pipette come straight up out of inoculum without flicking the side |  |  |
| Pipette tip is held just inside the mouth when inoculating |  |  |
| Pipette is held close to the surface of agar plates to minimize splashing |  |  |
| Good control over the liquid in the pipettes |  |  |
| Liquid is held at a steady level in pipettes(not allowed to bubble up in the bulb) |  |  |
| Care is taken to avoid dripping liquid from pipettes |  |  |
| The last bit of liquid is allowed to remain in the pipette (not completely expelled) |  |  |
| Pipettes are discarded correctly |  |  |
| Preventing cross-inoculation |  |  |
| Care is taken to insure tubes are in the right order and specimen numbers are double checked |  |  |
| After tube is inoculated it is moved to a new location |  |  |
| Final Score total |  | Please use the back for additional comments |

4 = excellent, confidence demonstrated.						Trainer initials________